IPv6 Network Configuration Workshop

Cisco Packet Tracer Tutorial

# Scenario

Your company has two sites in Telford and Wolverhampton. Each site requires a LAN, and the two sites must be connected via a WAN link. In this workshop, you will configure IPv6 addressing on routers and PCs, and implement RIPng (RIP next generation) as the routing protocol.

Why RIPng? The traditional RIP protocol does not support IPv6. RIPng was developed specifically for IPv6 networks while maintaining the same distance-vector metric as RIP. Unlike RIP for IPv4, RIPng runs as a process on each interface.

# IP Addressing Scheme

Use the following IPv6 addresses for this workshop:

| Device | Interface | IPv6 Address | Prefix |
| --- | --- | --- | --- |
| WOLVES Router | G0/0 | 2011:314:271:1::1 | /64 |
| WOLVES Router | S0/0/0 | FC00::12:1 | /112 |
| TELFORD Router | G0/0 | 2011:314:271:2::1 | /64 |
| TELFORD Router | S0/0/0 | FC00::12:2 | /112 |
| WOLVES PC | NIC | 2011:314:271:1::10 | /64 |
| TELFORD PC | NIC | 2011:314:271:2::10 | /64 |

# Part 1: Building the Network Topology

## 1.1 Open Cisco Packet Tracer

Step 1: Launch Cisco Packet Tracer on your computer.

Step 2: Create a new file (File > New) or use the default blank workspace.

## 1.2 Add the Routers

Step 3: In the bottom-left device panel, click on Network Devices (the icon that looks like a hub).

Step 4: Click on Routers in the submenu.

Step 5: Select the 2911 router (or any router with serial interfaces).

Step 6: Click on the workspace to place the first router. This will be the WOLVES router.

Step 7: Place a second 2911 router to the right. This will be the TELFORD router.

### Rename the Routers

Step 8: Click on the first router to open its configuration window.

Step 9: Go to the Config tab.

Step 10: In the Display Name field, type WOLVES and press Enter.

Step 11: Repeat for the second router, naming it TELFORD.

## 1.3 Add the Switches

Step 12: In the device panel, click on Switches.

Step 13: Select the 2960 switch.

Step 14: Place one switch below the WOLVES router.

Step 15: Place another switch below the TELFORD router.

## 1.4 Add the PCs

Step 16: In the device panel, click on End Devices.

Step 17: Select PC.

Step 18: Place one PC below the WOLVES switch (this is the WOLVES PC).

Step 19: Place another PC below the TELFORD switch (this is the TELFORD PC).

## 1.5 Cable the Network

Now connect all devices with the appropriate cables.

### Connect Routers to Switches (Straight-through cables)

Step 20: Click on the Connections icon (lightning bolt) in the device panel.

Step 21: Select Copper Straight-Through cable.

Step 22: Click on the WOLVES router, then select GigabitEthernet0/0.

Step 23: Click on the switch below it, then select any FastEthernet port (e.g., Fa0/1).

Step 24: Repeat to connect TELFORD router (G0/0) to its switch.

### Connect PCs to Switches (Straight-through cables)

Step 25: Using the same Copper Straight-Through cable:

Step 26: Connect WOLVES PC (FastEthernet0) to its switch (any available port, e.g., Fa0/2).

Step 27: Connect TELFORD PC (FastEthernet0) to its switch.

### Connect Routers Together (Serial cable for WAN link)

(Don’t forget to insert serial interface to each routers first. Choose HWIC-2T. Its simple and reliable. You do it via drag and drop. First turn the router off, the insert the module, then turn it back on)

Step 28: Click on Connections, then select Serial DCE cable.

Step 29: Click on WOLVES router and select Serial0/0/0.

Step 30: Click on TELFORD router and select Serial0/0/0.

Important The DCE end of the cable (WOLVES side) must provide the clock signal. We will configure the clock rate on the WOLVES router.

# Part 2: Configuring the WOLVES Router

Click on the WOLVES router, then go to the CLI tab. Type ‘no’, confirm and then Press Enter to start.

## 2.1 Enter Global Configuration Mode

Router> enable Router# configure terminal Router(config)# hostname WOLVES

## 2.2 Enable IPv6 Routing

This command enables the router to forward IPv6 packets:

WOLVES(config)# ipv6 unicast-routing

## 2.3 Configure the LAN Interface (G0/0)

Configure the interface that connects to the local network:

WOLVES(config)# interface GigabitEthernet0/0 WOLVES(config-if)# ipv6 address 2011:314:271:1::1/64 WOLVES(config-if)# ipv6 enable WOLVES(config-if)# ipv6 rip comms enable WOLVES(config-if)# no shutdown

Understanding the Commands ipv6 address - Assigns the IPv6 address to this interface ipv6 enable - Activates IPv6 processing on the interface ipv6 rip comms enable - Enables RIPng process named 'comms' on this interface no shutdown - Turns the interface on

## 2.4 Configure the WAN Interface (S0/0/0)

Configure the serial interface that connects to the TELFORD router:

WOLVES(config-if)# interface Serial0/0/0 WOLVES(config-if)# ipv6 address FC00::12:1/112 WOLVES(config-if)# ipv6 enable WOLVES(config-if)# ipv6 rip comms enable WOLVES(config-if)# clock rate 4000000 This command applies only to DCE interfaces WOLVES(config-if)# no shutdown

Why clock rate? The WOLVES router has the DCE end of the serial cable, so it must provide the clocking signal. The clock rate command sets the speed of the serial link. Important! - it does matter, which router did you connect to the DCE cable first! If it was WOLVES, then its the one that is DCE, if not vice versa. Only one router may become a clock manager. No two clock initiators are possible. So, keep in mind it all. You won’t be doing the same for the TELFORD router. If you forgot, just delete the wire (press del button, it moves into deleting mode, and esc to leave that mode) and then replug it first to WOLVES and then to TELFORD.

## 2.5 Exit and Save

WOLVES(config-if)# end WOLVES# copy running-config startup-config

Press Enter when prompted to confirm the filename.

# Part 3: Configuring the TELFORD Router

Click on the TELFORD router, then go to the CLI tab.

## 3.1 Basic Configuration

Router> enable Router# configure terminal Router(config)# hostname TELFORD TELFORD(config)# ipv6 unicast-routing

## 3.2 Configure the LAN Interface (G0/0)

TELFORD(config)# interface GigabitEthernet0/0 TELFORD(config-if)# ipv6 address 2011:314:271:2::1/64 TELFORD(config-if)# ipv6 enable TELFORD(config-if)# ipv6 rip comms enable TELFORD(config-if)# no shutdown

## 3.3 Configure the WAN Interface (S0/0/0)

TELFORD(config-if)# interface Serial0/0/0 TELFORD(config-if)# ipv6 address FC00::12:2/112 TELFORD(config-if)# ipv6 enable TELFORD(config-if)# ipv6 rip comms enable TELFORD(config-if)# no shutdown

Note TELFORD does not need the clock rate command because it has the DTE end of the serial cable. Verify, that you did actually connect the cable to WOLVES side first. It does matter!

## 3.4 Exit and Save

TELFORD(config-if)# end TELFORD# copy running-config startup-config

# Part 4: Configuring the PCs

## 4.1 Configure WOLVES PC

Step 1: Click on the WOLVES PC to open its configuration window.

Step 2: Go to the Desktop tab.

Step 3: Click on IP Configuration.

Step 4: Select the IPv6 Configuration section (you may need to scroll down).

Step 5: Enter the following settings:

| IPv6 Address | 2011:314:271:1::10 |
| --- | --- |
| Prefix Length | 64 |
| Default Gateway | 2011:314:271:1::1 |

## 4.2 Configure TELFORD PC

Step 6: Click on the TELFORD PC.

Step 7: Go to Desktop > IP Configuration.

Step 8: Enter the following settings:

| IPv6 Address | 2011:314:271:2::10 |
| --- | --- |
| Prefix Length | 64 |
| Default Gateway | 2011:314:271:2::1 |

# Part 5: Testing Connectivity

## 5.1 Ping from WOLVES PC to TELFORD PC

Step 1: Click on WOLVES PC.

Step 2: Go to Desktop > Command Prompt.

Step 3: Type the following command:

ping 2011:314:271:2::10

You should see successful replies. If not, wait a few seconds for RIPng to converge and try again.

## 5.2 Verify Router Interfaces

On each router, run these verification commands:

### Show interface status:

WOLVES# show ipv6 interface brief

This displays all interfaces with their IPv6 addresses and status (up/down).

### Show routing table:

WOLVES# show ipv6 route

This shows all known IPv6 routes. You should see:

- Connected routes (C) for directly connected networks

- Local routes (L) for the router's own addresses

- RIP routes (R) learned from the other router via RIPng

# Part 6: Experimenting with EUI-64

EUI-64 is a method for automatically generating the interface ID (last 64 bits) of an IPv6 address from the interface's MAC address.

## 6.1 Add EUI-64 Address to WOLVES Router

On the WOLVES router CLI:

WOLVES# configure terminal WOLVES(config)# interface GigabitEthernet0/0 WOLVES(config-if)# ipv6 address 2011:314:271:1::/64 eui-64 WOLVES(config-if)# end

## 6.2 Add EUI-64 Address to TELFORD Router

On the TELFORD router CLI:

TELFORD# configure terminal TELFORD(config)# interface GigabitEthernet0/0 TELFORD(config-if)# ipv6 address 2011:314:271:2::/64 eui-64 TELFORD(config-if)# end

## 6.3 Examine the Results

Now check the interface addresses:

WOLVES# show ipv6 interface brief

What do you notice? The interface now has TWO IPv6 addresses: 1. The manually configured address (2011:314:271:1::1) 2. The EUI-64 generated address (with a longer interface ID derived from the MAC address)

## 6.4 Testing Connectivity

From the WOLVES PC, try to ping both IPv6 addresses on your partner's router G0/0 interface. Were you able to ping both addresses?

## 6.5 Link-Local Addresses

Every IPv6 interface automatically generates a link-local address starting with FE80::. Check the show ipv6 interface brief output to find these addresses.

Try to ping your partner's link-local address from the TELFORD PC:

ping FE80::xxxx:xxxx:xxxx:xxxx

Were you successful? Link-local addresses are only valid within the local network segment. They cannot be routed across the WAN link. This ping should fail.

# Part 7: Research Topics for Your Portfolio

Write notes in your own words on the following topics. Use approximately 400 words total.

## Topic 1: IPv6 Addressing Scheme

Research and explain:

- Address scope: global unicast, unique local, link-local

- Address structure: the 128-bit format and hexadecimal notation

- Address types: unicast, multicast, anycast

## Topic 2: EUI-64 Address Construction

Research and explain how an EUI-64 interface identifier is constructed from a MAC address. Include:

- How FFFE is inserted into the MAC address

- Why the 7th bit (U/L bit) is flipped

- A worked example showing the conversion

# Appendix: Quick Reference

## Essential IPv6 Commands

| Command | Purpose |
| --- | --- |
| ipv6 unicast-routing | Enable IPv6 routing on the router |
| ipv6 address <addr>/<prefix> | Assign IPv6 address to interface |
| ipv6 enable | Enable IPv6 on the interface |
| ipv6 rip <name> enable | Enable RIPng on the interface |
| show ipv6 interface brief | Show IPv6 addresses and interface status |
| show ipv6 route | Display the IPv6 routing table |

## Network Diagram

Your completed network should look like this:

[WOLVES PC] [TELFORD PC] 2011:314:271:1::10 2011:314:271:2::10 ¦ ¦ [Switch] [Switch] ¦ ¦ G0/0 G0/0 [WOLVES]---S0/0/0---WAN---S0/0/0---[TELFORD] FC00::12:1 <--> FC00::12:2

End of Workshop