The answers to the research questions at the end of this document must submitted in the one document you are producing with your answers to all the research questions from every workshop.

Lab - Configuring VLANs and Trunking Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| S1 | VLAN 1 | 192.168.1.11 | 255.255.255.0 | N/A |
| S2 | VLAN 1 | 192.168.1.12 | 255.255.255.0 | N/A |
| PC-A | NIC | 192.168.10.3 | 255.255.255.0 | 192.168.10.1 |
| PC-B | NIC | 192.168.10.4 | 255.255.255.0 | 192.168.10.1 |
| PC-C | NIC | 192.168.20.3 | 255.255.255.0 | 192.168.20.1 |

Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Create VLANs and Assign Switch Ports

Part 3: Maintain VLAN Port Assignments and the VLAN Database

Part 4: Configure an 802.1Q Trunk between the Switches

Part 5: Delete the VLAN Database

Background / Scenario

Modern switches use virtual local-area networks (VLANs) to improve network performance by separating large Layer 2 broadcast domains into smaller ones. VLANs can also be used as a security measure by controlling which hosts can communicate. In general, VLANs make it easier to design a network to support the goals of an organization.

VLAN trunks are used to span VLANs across multiple devices. Trunks allow the traffic from multiple VLANS to travel over a single link, while keeping the VLAN identification and segmentation intact.

In this lab, you will create VLANs on both switches in the topology, assign VLANs to switch access ports, verify that VLANs are working as expected, and then create a VLAN trunk between the two switches to allow hosts in the same VLAN to communicate through the trunk, regardless of which switch the host is actually attached to.

Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings on the PC hosts and switches. You should use the Catalyst 2960 switches and workstations.

Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

Initialize and reload the switches as necessary.

Configure basic settings for each switch.

Console into the switch and enter global configuration mode.

Configure the host name as shown in the topology.

Configure the IP address listed in the Addressing Table for VLAN 1 on the switch.

S1(config)#int vlan 1

S1(config-if)#ip address 192.168.1.11 255.255.255.0

S1(config-if)no shutdown

- Repeat this for switch 2

Configure PC hosts.

Refer to the Addressing Table for PC host address information.

Test connectivity.

Verify that the PC hosts can ping one another.

Can PC-A ping PC-B?	_________

Can PC-A ping PC-C?	_________

Can PC-A ping S1?	_________

Can PC-B ping PC-C?	_________

Can PC-B ping S2?	_________

Can PC-C ping S2?	_________

Can S1 ping S2?	_________

If you answered no to any of the above questions, why were the pings unsuccessful?

_______________________________________________________________________________________

_______________________________________________________________________________________

Create VLANs and Assign Switch Ports

In Part 2, you will create student, faculty, and management VLANs on both switches. You will then assign the VLANs to the appropriate interface. The show vlan command is used to verify your configuration settings.

Create VLANs on the switches.

Create the VLANs on S1.

S1(config)# vlan 10

S1(config-vlan)# name Student

S1(config-vlan)# vlan 20

S1(config-vlan)# name Faculty

S1(config-vlan)# vlan 99

S1(config-vlan)# name Management

S1(config-vlan)# end

Create the same VLANs on S2.

Issue the show vlan command to view the list of VLANs on S1.

S1# show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6, Fa0/7, Fa0/8

Fa0/9, Fa0/10, Fa0/11, Fa0/12

Fa0/13, Fa0/14, Fa0/15, Fa0/16

Fa0/17, Fa0/18, Fa0/19, Fa0/20

Fa0/21, Fa0/22, Fa0/23, Fa0/24

Gi0/1, Gi0/2

10   Student                          active

20   Faculty                          active

99   Management                       active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1    enet  100001     1500  -      -      -        -    -        0      0

10   enet  100010     1500  -      -      -        -    -        0      0

20   enet  100020     1500  -      -      -        -    -        0      0

99   enet  100099     1500  -      -      -        -    -        0      0

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    -        0      0

1004 fdnet 101004     1500  -      -      -        ieee -        0      0

1005 trnet 101005     1500  -      -      -        ibm  -        0      0

Remote SPAN VLANs

------------------------------------------------------------------------------

Primary Secondary Type              Ports

------- --------- ----------------- ------------------------------------------

What is the default VLAN? ___________

What ports are assigned to the default VLAN?

____________________________________________________________________________________

Assign VLANs to the correct switch interfaces.

Assign VLANs to the interfaces on S1.

Assign PC-A to the Student VLAN.

S1(config)# interface f0/6

S1(config-if)# switchport mode access

S1(config-if)# switchport access vlan 10

Move the switch IP address VLAN 99.

S1(config)# interface vlan 1

S1(config-if)# no ip address

S1(config-if)# interface vlan 99

S1(config-if)# ip address 192.168.1.11 255.255.255.0

S1(config-if)# end

Issue the show vlan brief command and verify that the VLANs are assigned to the correct interfaces.

S1# show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/7, Fa0/8, Fa0/9

Fa0/10, Fa0/11, Fa0/12, Fa0/13

Fa0/14, Fa0/15, Fa0/16, Fa0/17

Fa0/18, Fa0/19, Fa0/20, Fa0/21

Fa0/22, Fa0/23, Fa0/24, Gi0/1

Gi0/2

10   Student                          active    Fa0/6

20   Faculty                          active

99   Management                       active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

Issue the show ip interface brief command.

What is the status of VLAN 99? Why?

____________________________________________________________________________________

Use the Topology to assign VLANs to the appropriate ports on S2.

Use the show vlan brief command to verify that the VLANs are assigned to the correct interfaces.

S2# show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6, Fa0/7, Fa0/8

Fa0/9, Fa0/10, Fa0/12, Fa0/13

Fa0/14, Fa0/15, Fa0/16, Fa0/17

Fa0/19, Fa0/20, Fa0/21, Fa0/22

Fa0/23, Fa0/24, Gi0/1, Gi0/2

10   Student                          active    Fa0/11

20   Faculty                          active    Fa0/18

99   Management                       active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

Is PC-A able to ping PC-B? Why?

____________________________________________________________________________________

Is S1 able to ping S2? Why?

____________________________________________________________________________________

Maintain VLAN Port Assignments and the VLAN Database

In Part 3, you will change VLAN assignments to ports and remove VLANs from the VLAN database.

Assign a VLAN to multiple interfaces.

On S1, assign interfaces F0/11 – 24 to VLAN 10.

S1(config)# interface range f0/11-24

S1(config-if-range)# switchport mode access

S1(config-if-range)# switchport access vlan 10

S1(config-if-range)# end

Issue the show vlan brief command to verify VLAN assignments.

Reassign F0/11 and F0/21 to VLAN 20.

Verify that VLAN assignments are correct.

Remove a VLAN assignment from an interface.

Use the no switchport access vlan command to remove the VLAN 10 assignment to F0/24.

S1(config)# interface f0/24

S1(config-if)# no switchport access vlan

S1(config-if)# end

Verify that the VLAN change was made.

Which VLAN is F0/24 now associated with?

____________________________________________________________________________________

Remove a VLAN ID from the VLAN database.

Add VLAN 30 to interface F0/24 without issuing the VLAN command.

S1(config)# interface f0/24

S1(config-if)# switchport access vlan 30

% Access VLAN does not exist. Creating vlan 30

Note: Current switch technology no longer requires that the vlan command be issued to add a VLAN to the database. By assigning an unknown VLAN to a port, the VLAN adds to the VLAN database.

Verify that the new VLAN is displayed in the VLAN table.

S1# show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6, Fa0/7, Fa0/8

Fa0/9, Fa0/10, Gi0/1, Gi0/2

10   Student                          active    Fa0/12, Fa0/13, Fa0/14, Fa0/15

Fa0/16, Fa0/17, Fa0/18, Fa0/19

Fa0/20, Fa0/22, Fa0/23

20   Faculty                          active    Fa0/11, Fa0/21

30   VLAN0030                         active    Fa0/24

99   Management                       active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

What is the default name of VLAN 30?

____________________________________________________________________________________

Use the no vlan 30 command to remove VLAN 30 from the VLAN database.

S1(config)# no vlan 30

S1(config)# end

Issue the show vlan brief command. F0/24 was assigned to VLAN 30.

After deleting VLAN 30, what VLAN is port F0/24 assigned to? What happens to the traffic destined to the host attached to F0/24?

____________________________________________________________________________________

S1# show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6, Fa0/7, Fa0/8

Fa0/9, Fa0/10, Gi0/1, Gi0/2

10   Student                          active    Fa0/12, Fa0/13, Fa0/14, Fa0/15

Fa0/16, Fa0/17, Fa0/18, Fa0/19

Fa0/20, Fa0/22, Fa0/23

20   Faculty                          active    Fa0/11, Fa0/21

99   Management                       active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

Can PC-A ping PC-B?	_________

Can PC-A ping PC-C?	_________

Can PC-B ping PC-C?	_________

Configure an 802.1Q Trunk Between the Switches

In Part 4, you will configure interface F0/1 to use the Dynamic Trunking Protocol (DTP) to allow it to negotiate the trunk mode. After this has been accomplished and verified, you will disable DTP on interface F0/1 and manually configure it as a trunk.

Use DTP to initiate trunking on F0/1.

The default DTP mode of a 2960 switch port is dynamic auto. This allows the interface to convert the link to a trunk if the neighboring interface is set to trunk or dynamic desirable mode.

Set F0/1 on S1 to negotiate trunk mode.

S1(config)# interface f0/1

S1(config-if)# switchport mode dynamic desirable

*Mar  1 05:07:28.746: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to down

*Mar  1 05:07:29.744: %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

S1(config-if)#

*Mar  1 05:07:32.772: %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

S1(config-if)#

*Mar  1 05:08:01.789: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan99, changed state to up

*Mar  1 05:08:01.797: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

You should also receive link status messages on S2.

S2#

*Mar  1 05:07:29.794: %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

S2#

*Mar  1 05:07:32.823: %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

S2#

*Mar  1 05:08:01.839: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan99, changed state to up

*Mar  1 05:08:01.850: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

Issue the show vlan brief command on S1 and S2. Interface F0/1 is no longer assigned to VLAN 1. Trunked interfaces are not listed in the VLAN table.

S1# show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/2, Fa0/3, Fa0/4, Fa0/5

Fa0/7, Fa0/8, Fa0/9, Fa0/10

Fa0/24, Gi0/1, Gi0/2

10   Student                          active    Fa0/6, Fa0/12, Fa0/13, Fa0/14

Fa0/15, Fa0/16, Fa0/17, Fa0/18

Fa0/19, Fa0/20, Fa0/22, Fa0/23

20   Faculty                          active    Fa0/11, Fa0/21

99   Management                       active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

Issue the show interfaces trunk command to view trunked interfaces. Notice that the mode on S1 is set to desirable, and the mode on S2 is set to auto.

S1# show interfaces trunk

Port        Mode             Encapsulation  Status        Native vlan

Fa0/1       desirable        802.1q         trunking      1

Port        Vlans allowed on trunk

Fa0/1       1-4094

Port        Vlans allowed and active in management domain

Fa0/1       1,10,20,99

Port        Vlans in spanning tree forwarding state and not pruned

Fa0/1       1,10,20,99

S2# show interfaces trunk

Port        Mode             Encapsulation  Status        Native vlan

Fa0/1       auto             802.1q         trunking      1

Port        Vlans allowed on trunk

Fa0/1       1-4094

Port        Vlans allowed and active in management domain

Fa0/1       1,10,20,99

Port        Vlans in spanning tree forwarding state and not pruned

Fa0/1       1,10,20,99

Note: By default, all VLANs are allowed on a trunk. The switchport trunk command allows you to control what VLANs have access to the trunk. For this lab, keep the default settings which allows all VLANs to traverse F0/1.

Verify that VLAN traffic is traveling over trunk interface F0/1.

Can S1 ping S2?	___________

Can PC-A ping PC-B?	___________

Can PC-A ping PC-C?	___________

Can PC-B ping PC-C?	___________

Can PC-A ping S1?	___________

Can PC-B ping S2?	___________

Can PC-C ping S2?	___________

If you answered no to any of the above questions, explain below.

____________________________________________________________________________________

____________________________________________________________________________________

Demonstrate to your tutor

Using the Table below and the same physical topology create 3 vlans( 15,25, 33 which correspond to Salaries, finance and staff) and demonstrate you have vlan trunked between the two switches

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| S1 | VLAN 1 | 192.168.1.1 | 255.255.255.0 | N/A |
| S2 | VLAN 1 | 192.168.1.2 | 255.255.255.0 | N/A |
| PC-A | NIC | 192.168.90.7 | 255.255.255.0 | 192.168.90.1 |
| PC-B | NIC | 192.168.90.8 | 255.255.255.0 | 192.168.90.1 |
| PC-C | NIC | 192.168.92.9 | 255.255.255.0 | 192.168.92.1 |

| Switch | Vlan | Ports |
| --- | --- | --- |
| 1 | 15 | Fa0/10 - 12 |
| 1 | 25 | Fa0/13 - 14 |
| 1 | 33 | Fa0/15 - 17 |
| 2 | 15 | Fa0/ 6 - 8 |
| 2 | 25 | Fa0/9-12 |
| 2 | 33 | Fa0/13 - 21 |

Issue the show interfaces trunk command to view trunked interfaces

Fa0/1

Fa0/1

Fa0/20

Fa0/17

Fa0/17

Demonstrate to your tutor that you have VLAN trunked between the two switches

Research Questions

Why may our switch have an IP address, given that switches make their switching decisions based on MAC addresses.

Why is vlan trunking used.

What is ‘router on a stick’ and how does it differ from the vlan exercise configuration you have undertook is this workshop