Workshop- QoS Concepts

By configuring the quality of service (QoS), you can provide preferential treatment to specific types of traffic at the expense of other traffic types. Without QoS, the device offers best-effort service for each packet, regardless of the packet contents or size. The device sends the packets without any assurance of reliability, delay bounds, or throughput.

Topology

Initial network configuration

- IP addresses on PCs, servers and routers

- Configure routers’ interfaces

- Routing on the routers using OSPF, refer to last week’s workshop how to do this if you need to

- Configure DNS record to match the web servers

- On the serial link configure a bandwidth of 1Mbps

Router1(config-if)#bandwidth 1000

Configure the QoS with the following requirements

On Router 1

Match traffic

Voice using NBAR (rtp)

Set DSCP to EF

HTTP using NBAR (http)

Set DSCP to AF31

ICMP using NBAR (icmp)

Set DSCP to AF11

Bind outbound on serial interface

Voice should get a priority bandwidth of 70% of the available bandwidth

HTTP should get minimum bandwidth of 60% of the remaining bandwidth

ICMP should get minimum bandwidth of 30% of the remaining bandwidth

Create class maps to classify traffic with NBAR

Router1(config)#class-map voice

Router1(config-cmap)#match protocol rtp

Router1(config-cmap)#exit

Router1(config)#class-map http

Router1(config-cmap)#match protocol http

Router1(config-cmap)#exit

Router1(config)#class-map icmp

Router1(config-cmap)#mat

Router1(config-cmap)#match protocol icmp

Router1(config-cmap)#

Create QoS policies to match the traffic classes created.

Create a policy named QOSMark (or any name of your choice) and inside the policy set some rules for the different classes

Router1(config)#policy-map QOSMark

Router1(config-pmap)#class voice

Router1(config-pmap-c)#set ip dscp ?

Explore the different values with their binary equivalent. Set the DSCP to EF and provide a priority for Voice traffic at 70% of the available bandwidth

Router1(config-pmap-c)#set ip dscp ef

Router1(config-pmap-c)#priority 70

Set the parameters for the other two traffic, HTTP and ICMP

Router1(config)#policy-map QOSMark

Router1(config-pmap)#class http

Router1(config-pmap-c)#set ip dscp af31

Router1(config-pmap-c)#bandwidth remaining percent 60

Router1(config-pmap)#class icmp

Router1(config-pmap-c)#set ip dscp af11

Router1(config-pmap-c)#bandwidth remaining percent 30

The next step is to bind the policy to the interface on the outbound direction and match it to the QoS policy created

Router1(config)#int s0/1/0

Router1(config-if)#service-policy output QoSMark

We now need to do something similar on Router 2 and configure QoS using DSCP.

Match traffic

Voice traffic using DSCP EF

Set IP precedence to 5

HTTP traffic using DSCP AF31

Set IP precedence to 3

ICMP traffic using DSCP AF11

Set IP precedence to 0

Bind inbound on serial interface

Create the classes and assign DSCP values, you are not using NBAR

Router2(config)#class-map voice

Router2(config-cmap)#match ip dscp EF

Router2(config-cmap)#exit

Router2(config)#class-map http

Router2(config-cmap)#match ip dscp af31

Router2(config-cmap)#exit

Router2(config)#class-map icmp

Router2(config-cmap)#match ip dscp af11

Router2(config-cmap)#

We are going to read the DSCP values on the packets received on the serial interface and remark the DSCP values

Create a new policy on Router 2 and call it QoSRemark (or any  other name of your choice)

Router2(config)#policy-map QoSRemark

Router2(config-pmap)#class voice

Router2(config-pmap-c)#set precedence ?

Explore the options. Note you can use the numeric values or the word. You can use the word critical or a value of 5

Router2(config-pmap-c)#set precedence critical

Router2(config-pmap-c)#exit

Router2(config-pmap)#class http

Router2(config-pmap-c)#set precedence flash

Router2(config-pmap-c)#exit

Router2(config-pmap)#class icmp

Router2(config-pmap-c)#set precedence routine

Check your configuration to see if you have the following settings:

Next bound the policy to the serial interface

Router2(config)#int s0/1/0

Router2(config-if)#service-policy input QoSRemark

Verify that settings are working as expected

Router1#show policy-map

What’s the summary of your QoS Settings?

Look at the statistics on the interface

Router1#show policy-map interface s0/1/0

Are there any matches for Voice, HTTP or ICMP traffic? Explain

Switch to simulation mode

Ping from PC1 to www.example1.com

Explore the DSCP values at the both sides of the two routers. Do they match your expected values in the settings?

Open a browser on PC2 and access www.example2.com. Observer the QoS values for HTTP traffic.