6CC009 Workshop

# Scenario

Your company has two sites in Telford and Wolverhampton, each requiring a LAN to be setup at the site and for WAN link to be enabled between them. In this scenario we will be using IPV6 on the routers and PCs.  The routing protocol RIP does not support IPV6 and so we will need to use one that has been developed with the same metric as RIP, but will support it called RIPng (RIP new generation, and IPV6 was originally called IP new generation).  The manner in which we set up the routers is somewhat different as RIPng runs a process on each interface that enacts the routing and creates the routing table.

S0/0/0

S0/0/0

TELFORD

WOLVES

FC00::12:0 /112

Ethernet switch

G0/0

G0/0

Wolves
2011:314:271:1::/64

Telford
2011:314:271:1::/64

.

Fig 1 Network set up for workshop 3.1

Student  1 configuring the WOLVES  router  :

Student 2 configuring the TELFORD router  :

# Working in pairs you are to decide who is TELFORD and other to WOLVES,

- Cable up your workstation to a switch,  then into the Ethernet port G0/0 of your router. Use a serial cable between the serial ports of the routers. This cable is directional and the end with the DCE marked on it must go into the S0/0/0 port of the WOLVES router

# Configuring the Routers

The WOLVES router requires the following configuration settings :

We need to switch on the routers ability to handle IPv6 routing.

WOLVES(config)# ipv6 unicast-routing

Now set an IPV6 address on the interface

## WOLVES(config)# Interface G0/0

WOLVES(config-if)# ipv6 address 2011:314:271:1::1/64

WOLVES(config-if)# ipv6 enable

Now we need to enable the IPV6 routing protocol on that interface and any interface that will be taking part in this.  This is unlike RIP using IPv4  which it enables the router to deal with RIP. The syntax is

ipv6 rip ‘process name’ enable

ipv6 rip <name> enable command enables the specified IPv6 RIP routing process on an interface. The process name is only significant within the router, and allows you to run more than one RIP process if you want to. Because it is only locally significant, every router can have a different RIP process name without conflict, although we generally don't recommend this, as it can become confusing to manage. We will call our process comms

WOLVES(config-if)# ipv6 rip comms enable

WOLVES(config-if)# no shutdown

WOLVES(config-if)# interface  s0/0/0

WOLVES(config-if)#  ipv6 address FC00::12:1/112

WOLVES(config-if)# ipv6 enable

WOLVES(config-if)# ipv6 rip comms enable

WOLVES(config-if)# clock rate 4000000

WOLVES(config-if)# no shutdown

The TELFORD router requires the following configuration settings :

TELFORD(config)# ipv6 unicast-routing

## TELFORD(config)# Interface G0/0

TELFORD(config-if)# ipv6 address 2011:314:271:2::1/64

TELFORD(config-if)# ipv6 enable

TELFORD(config-if)# ipv6 rip comms enable

TELFORD(config-if)# no shutdown

TELFORD(config-if)# interface  s0/0/0

TELFORD(config-if)# ipv6 address FC00::12:2/112

TELFORD(config-if)# ipv6 enable

TELFORD(config-if)# ipv6 rip comms enable

TELFORD(config-if)#no shutdown

Now set IPV6 addresses  on the Telford and Wolves PCs

| Wolves PC | 2011:314:271:1::2/64 |
| --- | --- |
| Telford PC | 2011:314:271:2::2/64 |

Check there is connectivity between the Telford pc and the Wolves PCs by using the IPV6 Ping command. Google  the command.

On each router issue the following commands

Telford# show ipv6 interface brief

This will show the addresses and state of the interfaces.

Telford# show IPV6 route

Will show the routing table

Now execute the following on your respective router

Wolves(config t)# int g0/0

Wolves (config - if)# ipv6 address 2011:314:271:1::/64 eui-64

Or

Telford(config t)# int g0/0

Telford(config - if)# ipv6 address 2011:314:271:2::/64 eui-64

Now examine the interface settings on each router

Wolves# show ipv6 interface brief

What do you notice?

What does the part of the command eui-64 do achieve ?

Ping the two IPV6 addresses that your colleague has set on their G0/0 interface.  Were you able to ping both addresses?

Paste a copy of your successful ping in the command window

Your colleagues PC will also have a link-local address starting with FE80::, ping their address.  Were you successful?  Why?

Research Topics FOR YOUR PORTFOLIO

Write notes in your words on the following topics using only 400 words.

IPV6 addressing scheme in terms of their scope, structure and type.

How a EUI-64 address is constructed.