COMPANION GUIDE

Workshop 3: VLANs and Trunking

A step-by-step explanation guide to accompany the official lab worksheet.

Read the concept explanations, follow the commands, and answer the questions with confidence.

Computer Networks Module – IDU / University of Wolverhampton

# Table of Contents

1. The Big Picture: What Are VLANs and Why Do We Need Them?

2. Key Vocabulary You Must Know

3. Understanding the Lab Topology

4. Part 1: Building the Network and Basic Configuration

5. Part 2: Creating VLANs and Assigning Ports

6. Part 3: Managing VLANs (Reassign, Remove, Delete)

7. Part 4: Configuring 802.1Q Trunk Between Switches

8. Part 5: Cleanup – Deleting the VLAN Database

9. Answering the Research Questions

10. Quick-Reference Command Cheat Sheet

11. Troubleshooting Common Mistakes

# 1. The Big Picture: What Are VLANs and Why Do We Need Them?

Core Concept Imagine a school with 200 computers all plugged into the same network. Every time any computer sends a broadcast (like an ARP request asking "who has this IP address?"), all 200 computers receive it. This wastes bandwidth and is a security risk – the finance department can see traffic from the student lab.

A VLAN (Virtual Local Area Network) solves this by logically dividing one physical switch into multiple separate networks. Computers in VLAN 10 cannot talk to computers in VLAN 20, even if they are plugged into the same physical switch. It is as if you had bought separate switches for each group – but without the extra cost.

### Three Reasons to Use VLANs

- Security: Students cannot sniff faculty traffic because the switch never forwards VLAN 10 frames to VLAN 20 ports.

- Performance: Broadcast domains are smaller, so less junk traffic reaches each host.

- Flexibility: You can group users by function (HR, Engineering, Guest) regardless of their physical location in the building.

### But Wait – What If VLANs Span Multiple Switches?

In our lab, Switch 1 (S1) and Switch 2 (S2) both have Student VLAN 10 ports. A student on S1 needs to reach a student on S2. The cable connecting S1 to S2 must carry traffic from all VLANs. This special cable is called a trunk link. The trunk uses 802.1Q tagging – a tiny label inserted into each Ethernet frame saying "I belong to VLAN 10" so the receiving switch knows where to deliver it.

# 2. Key Vocabulary You Must Know

Before touching a switch, make sure you understand these terms.

| Term | What It Means (Plain English) |
| --- | --- |
| VLAN | A virtual LAN created inside a switch. Each VLAN is its own broadcast domain. |
| Access Port | A switch port assigned to exactly one VLAN. Your PC plugs into an access port. |
| Trunk Port | A switch port that carries traffic for multiple VLANs between switches (or to a router). |
| 802.1Q Tag | A 4-byte label inserted into an Ethernet frame on a trunk. Contains the VLAN ID (1–4094). |
| Native VLAN | The one VLAN on a trunk that is sent untagged. By default, this is VLAN 1. |
| DTP | Dynamic Trunking Protocol – Cisco proprietary. Lets two switches auto-negotiate whether a link becomes a trunk. |
| switchport mode access | Forces the port to be an access port (one VLAN only). |
| switchport mode trunk | Forces the port to be a trunk (carries all VLANs). |
| dynamic desirable | The port actively tries to form a trunk with the other side. |
| dynamic auto | The port will become a trunk only if the other side asks. Default on 2960 switches. |
| show vlan brief | Displays which VLANs exist and which ports belong to each. |
| show interfaces trunk | Displays which ports are trunking, their mode, encapsulation, and allowed VLANs. |

# 3. Understanding the Lab Topology

The lab uses two Catalyst 2960 switches and three PCs. Here is the logical layout:

Check Your Addressing Table The PC IP addresses below follow the standard Cisco lab defaults. Always double-check against the Addressing Table image in your actual workshop handout – your instructor may have modified the IPs.

| Device | Interface | IP Address | Subnet Mask | VLAN Assignment |
| --- | --- | --- | --- | --- |
| PC-A | NIC | 192.168.1.1 | 255.255.255.0 | VLAN 10 (Student) |
| PC-B | NIC | 192.168.1.2 | 255.255.255.0 | VLAN 10 (Student) |
| PC-C | NIC | 192.168.1.3 | 255.255.255.0 | VLAN 20 (Faculty) |
| S1 | VLAN 99 | 192.168.1.11 | 255.255.255.0 | VLAN 99 (Mgmt) |
| S2 | VLAN 99 | 192.168.1.12 | 255.255.255.0 | VLAN 99 (Mgmt) |

Port Assignments:

| Switch | Port | Connected To | VLAN |
| --- | --- | --- | --- |
| S1 | F0/1 | S2 F0/1 | Trunk |
| S1 | F0/6 | PC-A | 10 (Student) |
| S2 | F0/11 | PC-B | 10 (Student) |
| S2 | F0/18 | PC-C | 20 (Faculty) |

Key Insight All PCs share the same IP subnet (192.168.1.0/24), but they are separated by VLANs. Even though their IPs "look" like they should reach each other, VLAN boundaries will block communication between VLAN 10 and VLAN 20 at Layer 2.

# 4. Part 1: Building the Network and Basic Configuration

### What You Are Doing and Why

Before creating VLANs, you need a working baseline. In this part you cable everything, give each switch an IP address (for remote management), configure the PCs, and verify that everything can ping everything. This proves the physical layer works before you start segmenting.

### Step-by-Step with Explanations

### 4.1 Cable the Network

Connect S1 F0/1 to S2 F0/1 with a crossover cable (or straight-through if using auto-MDIX switches). Connect PC-A to S1 F0/6, PC-B to S2 F0/11, PC-C to S2 F0/18.

### 4.2 Configure Switch Basic Settings

Console into S1. Enter the following commands:

Switch> enable

Switch# configure terminal

Switch(config)# hostname S1

S1(config)# interface vlan 1

S1(config-if)# ip address 192.168.1.11 255.255.255.0

S1(config-if)# no shutdown

S1(config-if)# exit

Why does a switch need an IP address? Switches are Layer 2 devices – they forward frames based on MAC addresses and do not need IP addresses to do their job. However, you need to manage the switch remotely (via SSH/Telnet). The IP on VLAN 1 (or later, VLAN 99) is a management interface so you can reach the switch over the network.

Repeat the same process for S2, using IP address 192.168.1.12.

### 4.3 Configure the PCs

Set the IP addresses on each PC as shown in the topology table. All PCs use subnet mask 255.255.255.0 and no default gateway is needed for this lab.

### 4.4 Test Connectivity

At this point, all devices are on VLAN 1 (the default). Every port on both switches belongs to VLAN 1, so all pings should succeed. Verify:

- PC-A to PC-B – should work (both on default VLAN 1)

- PC-A to PC-C – should work

- S1 to S2 – should work

If pings fail at this stage Check your cabling, verify IP addresses, and make sure you typed "no shutdown" on the VLAN 1 interface. Also confirm the PCs are in the same 192.168.1.0/24 subnet.

# 5. Part 2: Creating VLANs and Assigning Ports

### What You Are Doing and Why

Now you create three VLANs (10-Student, 20-Faculty, 99-Management) on both switches and assign specific ports to each. After this, connectivity between VLANs will break – and that is the whole point.

### 5.1 Create VLANs on S1

S1(config)# vlan 10

S1(config-vlan)# name Student

S1(config-vlan)# vlan 20

S1(config-vlan)# name Faculty

S1(config-vlan)# vlan 99

S1(config-vlan)# name Management

S1(config-vlan)# end

What just happened? You created three entries in the switch’s VLAN database (stored in flash memory as vlan.dat). These VLANs now exist but no ports belong to them yet – all ports are still on VLAN 1 (default).

Repeat the exact same commands on S2.

### 5.2 Verify with show vlan

Run show vlan brief on S1. You should see VLANs 10, 20, and 99 listed with no ports assigned. All ports still appear under VLAN 1.

Understanding the Output The "show vlan brief" output shows VLAN name, status, and which ports belong to it. Trunked ports will NOT appear here – they are shown by "show interfaces trunk" instead.

### 5.3 Assign Ports to VLANs

On S1 – assign PC-A (F0/6) to Student VLAN:

S1(config)# interface f0/6

S1(config-if)# switchport mode access

S1(config-if)# switchport access vlan 10

What do these two commands do? switchport mode access – tells the port it will serve a single VLAN only (not a trunk). switchport access vlan 10 – assigns that port to VLAN 10. Any device plugged into F0/6 is now in the Student VLAN.

On S2 – assign ports:

S2(config)# interface f0/11

S2(config-if)# switchport mode access

S2(config-if)# switchport access vlan 10

S2(config-if)# interface f0/18

S2(config-if)# switchport mode access

S2(config-if)# switchport access vlan 20

### 5.4 Move the Management IP to VLAN 99

The switch management IP was on VLAN 1. Now move it to VLAN 99 for proper segmentation.

On S1:

S1(config)# interface vlan 1

S1(config-if)# no ip address

S1(config-if)# interface vlan 99

S1(config-if)# ip address 192.168.1.11 255.255.255.0

S1(config-if)# end

Repeat on S2 with its own IP:

S2(config)# interface vlan 1

S2(config-if)# no ip address

S2(config-if)# interface vlan 99

S2(config-if)# ip address 192.168.1.12 255.255.255.0

S2(config-if)# end

Important After this step, VLAN 99 status will show "up/down" because no physical port is assigned to VLAN 99 yet. The management interface only comes fully up when at least one port in that VLAN is active. This is expected behavior.

### 5.5 Verify and Test

Can PC-A ping PC-B? No! Even though both are in VLAN 10, they are on different switches and the link between S1 and S2 (F0/1) is still a regular access port on VLAN 1. VLAN 10 traffic cannot cross it yet.

Can PC-A ping PC-C? No. They are in different VLANs (10 vs 20). VLANs block inter-VLAN traffic at Layer 2.

Can S1 ping S2? No. The management IPs are now on VLAN 99, but F0/1 is still on VLAN 1.

The Pattern Nothing can talk to anything across switches right now. This is because we have VLANs but no trunk yet. The trunk is configured in Part 4.

# 6. Part 3: Managing VLANs (Reassign, Remove, Delete)

This part teaches you how to manage VLANs after initial setup – bulk assignments, reassigning ports, and what happens when you delete a VLAN.

### 6.1 Assign Multiple Ports at Once

S1(config)# interface range f0/11-24

S1(config-if-range)# switchport mode access

S1(config-if-range)# switchport access vlan 10

The interface range command saves you from configuring 14 ports one by one. All ports F0/11 through F0/24 are now in VLAN 10.

### 6.2 Reassign a Port to a Different VLAN

S1(config)# interface f0/11

S1(config-if)# switchport access vlan 20

S1(config-if)# interface f0/21

S1(config-if)# switchport access vlan 20

This simply overwrites the previous VLAN assignment. F0/11 and F0/21 move from VLAN 10 to VLAN 20. No need to remove them first.

### 6.3 Remove a VLAN Assignment

S1(config)# interface f0/24

S1(config-if)# no switchport access vlan

The port returns to the default VLAN (VLAN 1).

### 6.4 What Happens When You Delete a VLAN?

First, assign F0/24 to a new VLAN 30 (the switch auto-creates it):

S1(config)# interface f0/24

S1(config-if)# switchport access vlan 30

Now delete VLAN 30 from the database:

S1(config)# no vlan 30

VLAN 30 Default Name When you created VLAN 30 by assigning it to a port (without using the "vlan 30" command first), the switch auto-named it "VLAN0030". This is the default naming convention: VLAN followed by a zero-padded 4-digit number. You can rename it with the "name" command inside vlan config mode.

Critical Concept: Orphaned Ports After deleting VLAN 30, port F0/24 is still configured as "access vlan 30" – but VLAN 30 no longer exists. The port becomes an orphan. It cannot send or receive any traffic. This is a common mistake in real networks. The fix: either reassign the port to an existing VLAN or recreate VLAN 30.

# 7. Part 4: Configuring 802.1Q Trunk Between Switches

### What You Are Doing and Why

This is the most important part of the lab. Right now, VLANs only work within each individual switch. The trunk link between S1 and S2 will carry tagged traffic for all VLANs, allowing Student-to-Student and Management-to-Management communication across switches.

### How 802.1Q Trunking Works

802.1Q Frame Tagging When a frame enters a trunk port, the switch inserts a 4-byte tag into the Ethernet header. This tag contains the VLAN ID (e.g., 10). The switch on the other end reads this tag, strips it, and delivers the frame to the correct VLAN. The native VLAN (default: VLAN 1) is special – its frames travel the trunk without a tag. Both sides must agree on the native VLAN.

### 7.1 Using DTP (Dynamic Trunking Protocol)

DTP is Cisco’s way of auto-negotiating trunk links. The 2960 default mode is dynamic auto (passive – waits for the other side to initiate). Setting one side to dynamic desirable (active) triggers negotiation.

DTP Negotiation Results:

| S1 Mode | S2 Mode | Result | Trunk? |
| --- | --- | --- | --- |
| dynamic desirable | dynamic auto | Trunk forms | Yes |
| dynamic desirable | dynamic desirable | Trunk forms | Yes |
| dynamic auto | dynamic auto | No trunk! | No |
| trunk | auto/desirable | Trunk forms | Yes |
| access | anything | No trunk | No |

Configure S1 F0/1 as dynamic desirable:

S1(config)# interface f0/1

S1(config-if)# switchport mode dynamic desirable

S2 F0/1 is still in its default mode (dynamic auto). Since desirable + auto = trunk, the trunk forms automatically. You will see log messages about the interfaces going down briefly and then coming back up.

### 7.2 Verify the Trunk

Run show interfaces trunk on both switches:

S1# show interfaces trunk

Port    Mode         Encapsulation  Status    Native vlan

Fa0/1   desirable    802.1q         trunking  1

Also run show vlan brief. Notice that F0/1 is no longer listed under any VLAN. Trunk ports are not shown in the VLAN table – they carry all VLANs.

### 7.3 Test Connectivity Again

Now test all the ping pairs from the original worksheet:

Can S1 ping S2? Yes! Both management IPs are on VLAN 99, and the trunk carries VLAN 99.

Can PC-A ping PC-B? Yes! Both are VLAN 10 and the trunk carries VLAN 10 between switches.

Can PC-A ping PC-C? No. PC-A is VLAN 10, PC-C is VLAN 20. Different VLANs still cannot communicate.

Can PC-B ping PC-C? No. Same reason – VLAN 10 vs VLAN 20.

Can PC-A ping S1? No. PC-A is VLAN 10, S1’s management IP is on VLAN 99.

Can PC-B ping S2? No. PC-B is VLAN 10, S2’s management IP is on VLAN 99.

Can PC-C ping S2? No. PC-C is VLAN 20, S2’s management IP is on VLAN 99.

The Golden Rule After trunking, devices in the SAME VLAN can communicate across switches. Devices in DIFFERENT VLANs still cannot – you would need a Layer 3 device (router) for that.

# 8. Part 5: Cleanup – Deleting the VLAN Database

When you are done with the lab, or if you want to start fresh, delete the VLAN database stored in flash:

S1# delete vlan.dat

S1# erase startup-config

S1# reload

The delete vlan.dat erases all VLAN definitions. The erase startup-config removes saved switch settings (hostname, IP addresses, etc.). After reload, the switch returns to full factory default. If you only delete vlan.dat without erasing startup-config, the switch will still have its hostname and other settings after reboot.

# 9. Answering the Research Questions

### Q1: Why does a switch have an IP address if it makes decisions based on MAC addresses?

Answer A switch does not need an IP address to forward frames – that happens purely at Layer 2 using MAC address tables. The IP address is for management purposes only: so that network administrators can access the switch remotely via SSH, Telnet, or a web interface. Without an IP, you could only configure the switch through the physical console cable. The IP is assigned to a virtual interface (SVI), typically VLAN 1 or a dedicated management VLAN like VLAN 99 in this lab.

### Q2: Why is VLAN trunking used?

Answer Trunking allows a single physical link between switches to carry traffic for multiple VLANs simultaneously. Without trunking, you would need a separate cable between switches for every VLAN – impractical and wasteful. 802.1Q trunk links tag each frame with its VLAN ID so the receiving switch can deliver it to the correct VLAN. This means you can have VLANs that span across an entire building (multiple switches) using just one inter-switch cable.

### Q3: What is "router on a stick" and how does it differ from this lab?

Answer In this lab, VLANs are completely isolated – VLAN 10 (Student) cannot talk to VLAN 20 (Faculty). A router on a stick adds a router connected to the switch via a trunk link. The router creates sub-interfaces (e.g., Gig0/0.10 for VLAN 10, Gig0/0.20 for VLAN 20), each with its own IP address acting as the default gateway for that VLAN. When a Student PC wants to reach a Faculty PC, the frame goes to the router, which routes it between VLANs and sends it back down the trunk to the correct VLAN. The key difference: this lab demonstrates Layer 2 segmentation only. Router on a stick adds Layer 3 inter-VLAN routing.

# 10. Quick-Reference Command Cheat Sheet

| Command | What It Does |
| --- | --- |
| vlan 10 | Creates VLAN 10 (enters VLAN config mode) |
| name Student | Names the VLAN (optional but recommended) |
| no vlan 10 | Deletes VLAN 10 from the database |
| switchport mode access | Sets port as access (single VLAN) |
| switchport access vlan 10 | Assigns port to VLAN 10 |
| no switchport access vlan | Returns port to default VLAN (1) |
| switchport mode trunk | Forces port into trunk mode |
| switchport mode dynamic desirable | Port actively negotiates trunk via DTP |
| interface range f0/11-24 | Selects multiple ports at once |
| show vlan brief | Shows VLAN list and port assignments |
| show interfaces trunk | Shows trunk ports and their settings |
| show interfaces f0/1 switchport | Shows detailed switchport config for a port |
| delete vlan.dat | Erases the entire VLAN database |

# 11. Troubleshooting Common Mistakes

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| PCs in same VLAN on different switches cannot ping | Trunk not configured between switches | Configure trunk on F0/1 (Part 4) |
| Port shows in no VLAN after deleting a VLAN | Orphaned port – still assigned to deleted VLAN | Reassign port to an existing VLAN |
| show vlan brief does not show F0/1 | F0/1 is a trunk port; trunks are not listed here | Use "show interfaces trunk" instead |
| VLAN 99 interface shows "up/down" | No physical port assigned to VLAN 99 is active | Assign a port to VLAN 99 or wait for trunk to form |
| Two auto ports will not trunk | auto + auto = no trunk (both passive) | Set at least one side to desirable or trunk |
| Pings between VLANs fail even with trunk | Expected! VLANs are Layer 2 isolation | Need a router ("router on a stick") for inter-VLAN routing |

End of Companion Guide

Good luck with the lab! Remember: if you can explain every command, you understand VLANs.